UNIVERSITY OF WAH (UOW)

DEPARTMENT OF COMPUTER SCIENCE

BSCS 3RD B

REG #: UW-24-CS-BS-088

SUBMITTED BY: SAADI SAFDAR

SUBMITTED TO: SIR. HASSAN SHAFAQAT

COURSE TITLE: COMPUTER NETWORKS

COURSE CODE: CS – 241 L

LAB REPORT # 13, 14 & 15

## Overview

This lab report contains five self-created practical tasks based on mandatory networking topics. Each task includes a unique instruction-based question, network design, IP addressing, device configuration, verification, and learning outcomes. All networks were implemented and tested in Cisco Packet Tracer.

Topics Covered:

- OSPF Dynamic Routing

- Wireless Router Connectivity

- BGP Dynamic Routing

- EIGRP Dynamic Routing

- Layer 3 NAT (Network Address Translation)

# 1. OSPF Dynamic Routing

## 1.1 Instruction-Type Question

Design and configure a three-router enterprise network using OSPF single-area routing. Each router must connect to a separate LAN. Implement dynamic routing using OSPF Area 0 and verify full end-to-end connectivity between all LAN hosts.

## 1.2 Reason for Choosing This Question

This question was selected to demonstrate practical implementation of OSPF, which is widely used in large-scale enterprise networks. The topology ensures multiple networks are involved, making dynamic routing necessary.

## 1.3 Network Topology

## 1.4 IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | Fa0 | 192.168.88.10 | 255.255.255.0 | 192.168.88.1 |
| Switch0 | VLAN 1 | 192.168.88.1 | 255.255.255.0 | — |
| Router0 | Gig0/0 | 192.168.88.1 | 255.255.255.0 | — |
| Router0 | Gig0/1 | 192.168.90.1 | 255.255.255.252 | — |
| Router1 | Gig0/0 | 192.168.90.2 | 255.255.255.252 | — |
| Router1 | Gig0/1 | 192.168.92.1 | 255.255.255.252 | — |
| Router2 | Gig0/0 | 192.168.92.2 | 255.255.255.252 | — |
| Router2 | Gig0/1 | 192.168.89.1 | 255.255.255.0 | — |
| Switch1 | VLAN 1 | 192.168.89.1 | 255.255.255.0 | — |
| PC1 | Fa0 | 192.168.89.10 | 255.255.255.0 | 192.168.89.1 |

## 1.5 Verification and Testing

## 1.6 Conclusion and Learning Outcome

This task improved understanding of dynamic routing using OSPF and demonstrated efficient route exchange without manual configuration.

# 2. Wireless Router Connectivity

## 2.1 Instruction-Type Question

Design a wireless network using a wireless router and multiple wireless clients. Configure secure Wi-Fi using WPA2 encryption and ensure internet access through DHCP and NAT.

## 2.2 Reason for Choosing This Question

Wireless networking is essential in modern networks. This task demonstrates real-world Wi-Fi configuration and security implementation.

## 2.3 Network Topology

## 2.4 IP Addressing Scheme

DHCP Range: 192.168.1.100 – 192.168.1.200
Router Gateway: 192.168.1.1

## 2.5 Configuration

- Set SSID: CampusNet

- Security: WPA2-PSK

- Password: cnlab123

- DHCP Enabled

## 2.6 Conclusion and Learning Outcome

This task provided hands-on experience with wireless configuration, security, and client connectivity.

# 3. BGP Dynamic Routing

## 3.1 Instruction-Type Question

Create a two-autonomous system network using BGP. Configure eBGP peering between two edge routers and verify route exchange between both AS networks.

## 3.2 Reason for Choosing This Question

BGP is the backbone routing protocol of the internet. This task demonstrates inter-AS communication.

## 3.3 Network Topology

## 3.4 IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC5 | Fa0 | 172.16.1.10 | 255.255.255.0 | 172.16.1.1 |
| Switch2 | VLAN 1 | 172.16.1.1 | 255.255.255.0 | — |
| Router8 (AS 65001) | Fa0/0 | 172.16.1.1 | 255.255.255.0 | — |
| Router8 (AS 65001) | Se0/3/0 | 192.0.2.1 | 255.255.255.252 | — |
| Router9 (AS 65002) | Se0/3/0 | 192.0.2.2 | 255.255.255.252 | — |
| Router9 (AS 65002) | Fa0/0 | 172.16.2.1 | 255.255.255.0 | — |
| Switch3 | VLAN 1 | 172.16.2.1 | 255.255.255.0 | — |
| PC6 | Fa0 | 172.16.2.10 | 255.255.255.0 | 172.16.2.1 |

## 3.5 Verification

## 3.6 Conclusion

This task improved understanding of inter-domain routing using BGP.

# 4. EIGRP Dynamic Routing

## 4.1 Instruction-Type Question

Design a four-router network and implement EIGRP routing. Verify route convergence and redundancy.

## 4.2 Reason

EIGRP provides fast convergence and efficient routing in Cisco environments.

## 4.3 Topology

## 4.4 IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC7 | Fa0 | 192.168.11.10 | 255.255.255.0 | 192.168.11.1 |
| Switch4 | VLAN 1 | 192.168.11.1 | 255.255.255.0 | — |
| Router10 | Fa0/0 | 192.168.11.1 | 255.255.255.0 | — |
| Router10 | Se2/0 | 192.168.55.1 | 255.255.255.252 | — |
| Router10 | Se3/0 | 192.168.88.2 | 255.255.255.252 | — |
| Router11 | Se2/0 | 192.168.23.1 | 255.255.255.252 | — |
| Router11 | Fa0/0 | 192.168.22.1 | 255.255.255.0 | — |
| Switch5 | VLAN 1 | 192.168.22.1 | 255.255.255.0 | — |
| PC8 | Fa0 | 192.168.22.10 | 255.255.255.0 | 192.168.22.1 |
| Router12 | Fa0/0 | 192.168.33.1 | 255.255.255.0 | — |
| Router12 | Se2/0 | 192.168.77.1 | 255.255.255.252 | — |
| Router12 | Se3/0 | 192.168.66.2 | 255.255.255.252 | — |
| Switch6 | VLAN 1 | 192.168.33.1 | 255.255.255.0 | — |
| PC9 | Fa0 | 192.168.33.10 | 255.255.255.0 | 192.168.33.1 |
| Router13 | Fa0/0 | 192.168.44.1 | 255.255.255.0 | — |
| Router13 | Se2/0 | 192.168.45.1 | 255.255.255.252 | — |
| Router13 | Se3/0 | 192.168.88.1 | 255.255.255.252 | — |
| Switch7 | VLAN 1 | 192.168.44.1 | 255.255.255.0 | — |
| PC10 | Fa0 | 192.168.44.10 | 255.255.255.0 | 192.168.44.1 |

## 4.5 Verification

## 4.6 Conclusion

This task enhanced understanding of advanced Cisco routing techniques.

# 5. Layer 3 NAT

## 5.1 Instruction-Type Question

Design a private enterprise network using Layer 3 NAT to allow internal hosts to access the public internet.

## 5.2 Reason

NAT is critical for IP conservation and internet connectivity.

## 5.3 Topology

## 5.4 IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC11 | Fa0 | 192.168.50.10 | 255.255.255.0 | 192.168.50.1 |
| PC12 | Fa0 | 192.168.50.11 | 255.255.255.0 | 192.168.50.1 |
| Switch8 | VLAN 1 | 192.168.50.1 | 255.255.255.0 | — |
| Router14 (NAT) | Fa0/0 | 192.168.50.1 | 255.255.255.0 | — |
| Router14 (NAT) | Se0/3/0 | 203.0.113.1 | 255.255.255.252 | — |
| Router15 (ISP) | Se0/3/0 | 203.0.113.2 | 255.255.255.252 | — |
| Router15 (ISP) | Fa0/0 | 198.51.100.1 | 255.255.255.0 | — |
| Server1 | Fa0 | 198.51.100.10 | 255.255.255.0 | 198.51.100.1 |

## 5.5 Configuration

R1:

R2:

IP Summary:

## 5.6 Verification

## 5.7 Conclusion and Learning Outcome

This task demonstrated real-world NAT configuration and public-private IP translation.

# Final Conclusion

This lab strengthened practical knowledge of routing protocols, wireless networking, and NAT. It improved hands-on skills in designing, configuring, verifying, and troubleshooting real-world networks using Cisco Packet Tracer.