UNIVERSITY OF WAH (UOW)

DEPARTMENT OF COMPUTER SCIENCE

BSCS 3RD B

REG #: UW-24-CS-BS-088

SUBMITTED BY: SAADI SAFDAR

SUBMITTED TO: SIR. HASSAN SHAFAQAT

COURSE TITLE: COMPUTER NETWORKS

COURSE CODE: CS – 241 L

FINAL LAB SESSIONAL

Q1. A family home has multiple smartphones and laptops that need secure wireless

access while a desktop computer is connected via cable; DESIGN a wireless network

using 2 wireless router, 3 wireless devices, and 1 wired PC, configure wireless

security and IP addressing, and confirm that all devices can communicate successfully

within the network.

R1:

R2:

PC:

Q2: A multinational company connects its headquarters network to a partner

organization’s network, where both use separate routing domains; APPLY BGP

dynamic routing by designing a topology with 4 routers (2 per Autonomous System),

2 switches, and 2 PCs, configure eBGP between the AS networks, and verify that

routes are exchanged correctly.

Topology:

IP Addressing:

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC2 | Fa0 | 192.168.189.2 | 255.255.255.0 | 192.168.189.1 |
| Router0 | Fa0/0 | 192.168.189.1 | 255.255.255.0 | N/A |
| Router0 | Se2/0 | 192.168.191.1 | 255.255.255.0 | N/A |
| Router1 | Se3/0 | 192.168.191.2 | 255.255.255.0 | N/A |
| Router1 | Se2/0 | 192.168.192.1 | 255.255.255.0 | N/A |
| Router2 | Se3/0 | 192.168.192.2 | 255.255.255.0 | N/A |
| Router2 | Se2/0 | 192.168.193.1 | 255.255.255.0 | N/A |
| Router3 | Se3/0 | 192.168.193.2 | 255.255.255.0 | N/A |
| Router3 | Fa0/0 | 192.168.190.1 | 255.255.255.0 | N/A |
| PC3 | Fa0 | 192.168.190.2 | 255.255.255.0 | 192.168.190.1 |

CLI for routers:

Router 0

Bash

en

conf t

ip route 0.0.0.0 0.0.0.0 192.168.191.2

### Router 1

Bash

en

conf t

ip route 192.168.189.0 255.255.255.0 192.168.191.1

router bgp 100

neighbor 192.168.192.2 remote-as 200

network 192.168.189.0 mask 255.255.255.0

network 192.168.191.0 mask 255.255.255.0

### Router 2

Bash

en

conf t

ip route 192.168.190.0 255.255.255.0 192.168.193.2

router bgp 200

neighbor 192.168.192.1 remote-as 100

network 192.168.190.0 mask 255.255.255.0

network 192.168.193.0 mask 255.255.255.0

### Router 3

Bash

en

conf t

ip route 0.0.0.0 0.0.0.0 192.168.193.1

Results:

Q3: A logistics company has three warehouses connected through routers and needs

fast route convergence to avoid delivery delays; ANALYZE the network and

configure EIGRP using 3 routers, 3 switches, and 3 PCs, ensure all routers use the

same EIGRP autonomous system number, disable auto-summary, and verify end-to-

end connectivity.

IP  Addressing:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC2 | Fa0 | 192.168.189.2 | 255.255.255.0 |
| Router0 | Fa0/0 | 192.168.189.1 | 255.255.255.0 |
| Router0 | Se2/0 | 192.168.191.1 | 255.255.255.0 |
| Router1 | Se3/0 | 192.168.191.2 | 255.255.255.0 |
| Router1 | Se2/0 | 192.168.192.1 | 255.255.255.0 |
| Router2 | Se3/0 | 192.168.192.2 | 255.255.255.0 |
| Router2 | Se2/0 | 192.168.193.1 | 255.255.255.0 |
| Router3 | Se3/0 | 192.168.193.2 | 255.255.255.0 |
| Router3 | Fa0/0 | 192.168.190.1 | 255.255.255.0 |
| PC3 | Fa0 | 192.168.190.2 | 255.255.255.0 |

CLI for routers:

On Router 4:

Bash

en

conf t

router eigrp 88

no auto-summary

network 192.168.177.0

network 192.168.180.0

network 192.168.182.0

On Router 5:

Bash

en

conf t

router eigrp 88

no auto-summary

network 192.168.178.0

network 192.168.180.0

network 192.168.181.0

On Router 6:

Bash

en

conf t

router eigrp 88

no auto-summary

network 192.168.179.0

network 192.168.181.0

network 192.168.182.0

Results: